HEALTH, SAFETY & ENVIRONMENT MANAGEMENT SYSTEM

DAN Construction Safety Manual (DCSM)

Road Works

Document Control

| Document No. | 01-000001-000000-DAN-HSE-PRC-000037 | Revision | R01 |
| --- | --- | --- | --- |
| Issue Date | June 2026 | Classification | Controlled — Internal |
| Document Owner | HSE Department | Page Format | US Letter |

Table of Contents

1. Purpose

2. Scope

3. References

4. Definitions

5. Deliverables

6. General Requirements

7. Flagging and Traffic Control

8. Night Work

9. Reinstatement and Completion

10. Records / Related Documents

Revision History

# 1. Purpose

This document describes the minimum safety requirements for the construction and repair of roadways for DAN projects, facilities and communities.

# 2. Scope

This document applies to all roadwork on DAN projects — traffic control, signage, flagging, excavation protection, dust control, night work and reinstatement — and to the coordination of road closures and diversions.

# 3. References

- Saudi Arabian Government — Ministry of Communications (Transport) requirements

- Manual on Uniform Traffic Control Devices (MUTCD)

- DCSM — Signage, Barrication and Traffic Control; DCSM — Excavations, Shoring and Backfilling

# 4. Definitions

| Term | Definition |
| --- | --- |
| Traffic Control Device | A barrier, sign, light, cone or marker used to warn, guide or regulate traffic through a work area. |
| Flagman | A trained, high-visibility person who controls traffic through or around the roadwork area. |
| Diversion / Detour | A temporary re-routing of traffic around a closed or restricted road section. |

# 5. Deliverables

- Road Work Method Statement (for DAN approval prior to starting road work)

- Road Work Risk Assessment (for DAN approval prior to starting road work)

# 6. General Requirements

- Traffic control devices (barriers, lights, cones, etc.) shall conform to local traffic-authority requirements; hazard-warning devices shall be installed before roadwork starts and promptly removed on completion.

- Placement of traffic-control and construction signs along the work area shall be approved by the DAN Representative; spacing shall be adjusted to suit community or facility street systems.

- Signs shall be maintained clean and legible and made of reflective sheeting.

- Dust-producing activities shall be minimised; where dust cannot be avoided and presents a hazard to traffic or a nuisance to nearby facilities/residences, effective dust control (e.g. wetting of topsoil) shall be implemented.

- Excavations shall be barricaded or otherwise protected for pedestrian safety and shored as necessary to prevent sidewall cave-ins, especially from nearby traffic (see the Excavations chapter).

# 7. Flagging and Traffic Control

- Flagmen shall be posted at entrance and exit barricades, equipped with high-visibility (reflective) vests, reflective "stop"/"go" signs or flags, and/or radio communications.

- Entrance and exit flagmen shall remain within sight of each other; where conditions do not permit this, additional flagmen shall be positioned (where safe) so they can see and be seen by the others to safely control traffic.

- Excavations and detours for roadways/streets shall be in accordance with the Authorities and DAN; road closures and/or diversions shall be coordinated with adjacent projects and authorised via permit from the DAN Representative.

# 8. Night Work

- Roadwork areas shall be illuminated at night; flagmen shall wear high-visibility (reflective) vests and gloves, be stationed in illuminated areas, and use reflective signs or flags.

- Flashing beacons and reflective signage shall be used on excavations next to roadways and pedestrian routes used at night.

# 9. Reinstatement and Completion

- Surfaces of highways, streets and sidewalks shall be level and in good condition before barricades and markers are removed.

- Permanent road signs and striping shall be restored before the roadway is returned to service.

- On completion, all excess materials, spoils, debris and traffic-control signs shall be immediately removed from the area.

# 10. Records / Related Documents

Approved method statements, risk assessments and road-closure/diversion permits shall be maintained on-site for the duration of the works and archived at project close.

- DCSM — Signage, Barrication and Traffic Control

- DCSM — Excavations, Shoring and Backfilling

- DCSM — Motor Vehicles and Equipment

# Revision History

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| R00 | June 2026 | Initial issue. Restructured to the DAN integrated HSSE management-system template and re-branded in accordance with the DAN Brand Book. | Project Director |
| R01 | June 2026 | Re-coded to the updated DAN document-numbering convention (01-000001-000000-DAN-{discipline}-{type}-{seq}). | Project Director |