HEALTH, SAFETY & ENVIRONMENT MANAGEMENT SYSTEM

DAN Construction Safety Manual (DCSM)

Night Work

Document Control

| Document No. | 01-000001-000000-DAN-HSE-PRC-000048 | Revision | R01 |
| --- | --- | --- | --- |
| Issue Date | June 2026 | Classification | Controlled — Internal |
| Document Owner | HSE Department | Page Format | US Letter |

Table of Contents

1. Purpose

2. Scope

3. References

4. Definitions

5. Deliverables

6. Risk Assessment and Night Works Plan

7. General Requirements and Fatigue Management

8. Area Lighting

9. Light Placement, Bulbs and Fixtures

10. Personal Protective Equipment

11. Records / Related Documents

Revision History

# 1. Purpose

This document provides the requirements and responsibilities for any work activity carried out during the hours of darkness, in areas where general illumination is required, on DAN projects.

# 2. Scope

This document applies to all night work on DAN projects — planning and risk assessment, fatigue management, area lighting, light fixtures and PPE.

# 3. References

- 29 CFR 1926.56 — Illumination

- DCSM — Electrical Safety; DCSM — Permit to Work; DCSM — Signage, Barrication and Traffic Control

- DCSM — SH&E Risk Management; DCSM — Personal Protective Equipment

# 4. Definitions

| Term | Definition |
| --- | --- |
| Night Works Plan | An approved plan defining the scope, controls and resources for work performed during darkness. |
| SWMS | Safe Work Method Statement. |
| Sufficient Sleep | At least a continuous 8-hour sleep period in the previous 24 hours. |
| Lux | The SI unit of illuminance used to specify and measure lighting levels. |

# 5. Deliverables

- Night Work Procedures

- Safe Work Method Statement (SWMS)

- Night Works Plan; illumination-level records

# 6. Risk Assessment and Night Works Plan

- Risk assessments shall be conducted at the planning stage and completed before any night work commences.

- A Night Works Plan shall be submitted to the SH&E Representative and Superintendent for approval, containing the scope, location, resources, emergency response, risk assessment/AHA, night-shift-specific training, permit-to-work, employee welfare, traffic and fatigue management, specific PPE, transportation and key contacts.

- A formal job briefing shall be performed before work, highlighting hazards and controls; the approved plan shall be available on-site; additional work outside the scope requires Superintendent agreement and a SWMS reviewed by the SH&E Representative and approved by the Project Manager; revisions follow the same approval route.

Note:  The following activities are PROHIBITED on night shifts: removal of flooring or grid mesh; operating vehicles/equipment without headlights; and any work requiring a man-cage.

# 7. General Requirements and Fatigue Management

- Every effort shall be made to minimise night-working hours; adequate barriers shall surround hazard areas, and all hazards shall be clearly lit.

- Fatigue management is the responsibility of supervisors and each individual; personnel shall arrive fit for work, having had "sufficient sleep" (at least 8 continuous hours in the previous 24), and shall recognise fatigue in themselves and others.

- Personnel shall immediately stop operating machinery or mobile equipment and notify the supervisor if they did not get sufficient sleep, feel fatigued/drowsy, fall asleep, or take medication causing drowsiness; supervisors shall track each crew member's fatigue status at shift start and during the shift.

# 8. Area Lighting

- Area lighting shall provide illumination sufficient for personnel and equipment to move and work safely, based on the activity, area and crew size; all installed lighting shall be inspected, meet project and local code, and be regularly maintained.

- Illumination shall be measured with a calibrated lux light meter at setup and periodically, with records maintained; exits, pathways, muster points and ladder access shall be clearly illuminated and marked.

- Lights shall be positioned to avoid glare; tower lighting shall illuminate all work areas; confined spaces shall be lit; back-up lighting shall be provided against power failure; portable generators shall meet the Electrical Safety chapter; supporting cables shall be routed clear of hazards and fixtures secured against falling.

Minimum lighting requirements by work activity:

| Work Activity | Lux | Foot-candles |
| --- | --- | --- |
| Interior movement only | 40 | 4.0 |
| Handling material | 80 | 8.0 |
| General rough work / interior working places | 160 | 16.0 |
| Interior reinforcing / concrete placement | 160 | 16.0 |
| Bricklaying / office lighting | 320 | 32.0 |
| Bench work / plastering / interior workshops | 400 | 40.0 |
| Drawing boards | 600 | 60.0 |

# 9. Light Placement, Bulbs and Fixtures

- Layouts shall illuminate all areas from at least two directions to prevent shadows; lighting shall be mounted on poles/towers and spaced for optimal distribution, positioned so personnel do not encounter fixtures during work.

- Cables and materials shall be positioned to prevent trip/fall hazards; portable cord trees, stands and poles shall suspend cords and lighting; light stringers and cords shall not be hung from scaffolding, and attachments to structural steel/piping shall be indirect using non-conductive ties.

- Conventional tungsten or tungsten-halogen lighting should be used where possible; high/low-pressure sodium should be avoided (fire risk and specialised disposal); MSDS shall be checked for each bulb type, and bulbs protected from breakage with lamp guards/cages, with broken bulbs replaced promptly.

- Lighting fixtures shall be certified to the local electrical code, securely installed, with supply cords per good wiring practice; metal poles and metal-case sockets shall be earthed/grounded with the circuit fitted with residual-current devices; temporary lighting strings shall use non-conductive sockets moulded to the conductor insulation.

# 10. Personal Protective Equipment

- PPE for night work or dark areas shall be identified on the SWMS/AHA and selected for the expected hazards including low-light conditions.

- Safety glasses shall be clear or light-enhancing; shirts/jackets shall be long-sleeved with 50 mm reflective tape on chest, back and sleeves; trousers shall have 50 mm reflective tape around the knees; all personnel shall carry a ready flashlight; crews out of direct verbal range of the supervisor shall have radio or telephone communication.

Note:  Flashlights MAY NOT be substituted for area lighting.

# 11. Records / Related Documents

Night Works Plans, SWMS, illumination-level records and training records shall be maintained on-site and archived at project close.

- DCSM — Electrical Safety

- DCSM — Permit to Work

- DCSM — Signage, Barrication and Traffic Control

- DCSM — Personal Protective Equipment

# Revision History

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| R00 | June 2026 | Initial issue. Restructured to the DAN integrated HSSE management-system template and re-branded in accordance with the DAN Brand Book. | Project Director |
| R01 | June 2026 | Re-coded to the updated DAN document-numbering convention (01-000001-000000-DAN-{discipline}-{type}-{seq}). | Project Director |