HEALTH, SAFETY & ENVIRONMENT MANAGEMENT SYSTEM

DAN Construction Safety Manual (DCSM)

Underground Construction

Document Control

| Document No. | 01-000001-000000-DAN-HSE-PRC-000030 | Revision | R01 |
| --- | --- | --- | --- |
| Issue Date | June 2026 | Classification | Controlled — Internal |
| Document Owner | HSE Department | Page Format | US Letter |

Table of Contents

1. Purpose

2. Scope

3. References

4. Deliverables

5. Project Underground Construction Plan

6. General Requirements

7. Communication and Emergency Provisions

7.1 Rescue Teams

8. Ground Support and Drilling

9. Air-Quality Monitoring

10. Ventilation and Fire Prevention

11. Hauling and Hoisting

12. Training

13. Records

14. Related Documents

Revision History

# 1. Purpose

This document details the requirements for the construction of underground tunnels, shafts, chambers and passageways, and for cut-and-cover excavations physically connected to ongoing underground operations and covered so as to create conditions characteristic of underground construction.

# 2. Scope

This document applies to all underground construction on DAN projects, including ground support, air monitoring, ventilation, fire prevention, hauling, hoisting and emergency rescue.

# 3. References

- 29 CFR 1926 Subpart S (Underground Construction, Caissons, Cofferdams and Compressed Air)

- DCSM — Excavations, Shoring and Backfilling; DCSM — Emergency Planning and Response

- DCSM — Fire Protection and Prevention; DCSM — Cranes and Hoists

# 4. Deliverables

- Project Underground Construction Plan

- Emergency Rescue Plan

# 5. Project Underground Construction Plan

A Competent Person in charge of the work develops the Plan (included in the Project Safety Plan or as a separate document). As a minimum it shall address the responsible and competent persons, AHAs and toolbox talks, controlled access/egress and check-in/check-out, communication, inspection and testing, air monitoring (substances and frequency), signage, training, emergency rescue, fire protection, ventilation, flood prevention and compressed-air work.

# 6. General Requirements

- Personnel shall be instructed in recognising and avoiding AHA hazards; oncoming shifts shall be informed of hazardous occurrences (gas liberation, equipment failure, slides, cave-ins, flooding, fire, explosion).

- Personnel in wet areas shall wear rubber boots and rain gear; first-aid stations shall be provided at each portal/shaft or within 8 km (5 miles) and 15 minutes travel time.

- Internal-combustion engines (except diesel on mobile equipment) are prohibited underground; a qualified electrician shall perform electrical work and trained welders shall perform hot work.

- Access and egress shall protect personnel from being struck by excavators, haulage machines and mobile equipment; access to all openings shall be controlled to prevent unauthorised entry.

# 7. Communication and Emergency Provisions

- Where unassisted voice communication is ineffective, a power-assisted means shall connect the work face, shaft bottom and surface; two means of communication (one voice) shall be provided in shafts used for access or hoisting, tested on entry each shift.

- An Emergency Rescue Plan shall be included and drilled; emergency equipment shall be within 15 minutes of each portal/shaft; all personnel shall have immediately available self-rescue respirators; a check-in/check-out procedure and a dedicated aboveground Designated Person shall maintain an accurate count of persons underground.

## 7.1 Rescue Teams

- Where 25 or more personnel work underground at one time, two 5-person rescue teams shall be arranged — one on-site or within one hour, the other within two hours (on-call); where fewer than 25, at least one 5-person team within ½ hour.

- Rescue-team members shall be qualified and recertified annually in rescue procedures, breathing apparatus and firefighting, and shall practise donning SCBA monthly where flammable/noxious gases may be encountered.

# 8. Ground Support and Drilling

- Ground shall be stabilised by shoring, filling, barricading and scaling; shafts or wells over 1.5 m (5 ft) deep entered by personnel shall be supported by casing, concrete pipe, timber or solid rock; no one shall work in unsupported sections.

- Conventional excavation shall not extend more than 24 in (60 cm) ahead of ground supports (48 in for continuous mining machines); voids behind supports shall be filled, blocked or braced.

- A Competent Person shall inspect drilling equipment before each use; personnel shall not be on a drill mast while the bit operates; jumbo decks over 10 ft high shall have guard-railed stairs wide enough for two persons.

# 9. Air-Quality Monitoring

A qualified Competent Person shall conduct air monitoring. Oxygen shall be tested before contaminants (normal range 20.5–22%). The atmosphere shall be tested for carbon monoxide, nitrogen dioxide, hydrogen sulphide and other contaminants as often as necessary to keep below the PEL. Key flammable-gas thresholds:

| Condition | Required action |
| --- | --- |
| H₂S ≥ 5 ppm | Test at start and midpoint of each shift until < 5 ppm for 3 consecutive days |
| H₂S > 10 ppm | Continuous monitoring; notify personnel |
| H₂S 20 ppm | Visual and audible alarm — respirators, increased ventilation or evacuation |
| Flammable gas > 5% LEL | Increase ventilation or otherwise control the gas concentration |
| Flammable gas > 10% LEL (hot work) | Suspend hot work until reduced below 10% LEL |
| Flammable gas > 20% LEL | Withdraw all non-essential personnel aboveground; cut power to the area |

# 10. Ventilation and Fire Prevention

- Mechanical ventilation shall supply at least 200 ft³ (5.7 m³) of fresh air per minute per person, with a linear velocity of at least 30 ft/min where dust or gases are produced; airflow shall be reversible and exhaust smoke/fumes after blasting.

- Open flames are prohibited except for permitted hot work; gasoline shall not be carried, stored or used underground; no more than a 24-hour supply of diesel, fuel gas and oxygen shall be stored underground.

- Fire extinguishers shall be provided at each portal/shaft entry, within 100 ft of the advancing face, at combustible-material locations, and at conveyor head/tail pulleys; underground structures within 100 ft of an opening shall have a one-hour fire-resistance rating.

# 11. Hauling and Hoisting

- Powered haulage equipment shall meet OSHA/ANSI specifications with audible and visual warnings, inspected before each shift; personnel shall not ride conveyors (endless belt-type man-lifts are prohibited).

- Cranes and hoists shall comply with the Cranes and Hoists chapter; loads shall be secured against shifting into the shaft; a warning light shall flash when a load is above or moving in the shaft; where a hoistway is not fully enclosed, conveyances shall stop at least 15 ft above the bottom until the Signalman directs the Operator.

# 12. Training

Personnel shall be trained at initial assignment, with supervisors identifying additional needs during method-statement and AHA development. Training covers air monitoring, PPE, ventilation, explosives, illumination, fire prevention, communications, emergency procedures (flood control, evacuation, check-in/check-out) and mechanical equipment. Rescue-team members are trained in jobsite conditions at least annually, with retraining on job/equipment changes.

# 13. Records

Training records, air-monitoring records, inspection forms and the Underground Construction and Emergency Rescue Plans shall be maintained on-site and archived at project close.

# 14. Related Documents

- DCSM — Excavations, Shoring and Backfilling

- DCSM — Emergency Planning and Response

- DCSM — Fire Protection and Prevention

- Project Underground Construction Plan; Emergency Rescue Plan

# Revision History

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| R00 | June 2026 | Initial issue. Restructured to the DAN integrated HSSE management-system template and re-branded in accordance with the DAN Brand Book. | Project Director |
| R01 | June 2026 | Re-coded to the updated DAN document-numbering convention (01-000001-000000-DAN-{discipline}-{type}-{seq}). | Project Director |