HEALTH, SAFETY & ENVIRONMENT MANAGEMENT SYSTEM

DAN Construction Safety Manual (DCSM)

Painting and Coating

Document Control

| Document No. | 01-000001-000000-DAN-HSE-PRC-000047 | Revision | R01 |
| --- | --- | --- | --- |
| Issue Date | June 2026 | Classification | Controlled — Internal |
| Document Owner | HSE Department | Page Format | US Letter |

Table of Contents

1. Purpose

2. Scope

3. References

4. Definitions

5. Deliverables

6. Flammability Hazards

7. Health Hazards

8. Surface and Worksite Preparation

9. Application

10. Tarring Operations

11. Records / Related Documents

Revision History

# 1. Purpose

This document describes the minimum safety requirements for spray painting and coating on DAN projects, covering flammability and health hazards, surface preparation, application and tarring operations.

# 2. Scope

This document applies to all painting and coating activities on DAN projects, including surface preparation, spray and airless application, the use of solvents and cleaners, and hot-tar operations.

# 3. References

- NFPA 30 — Flammable and Combustible Liquids Code; UL 844

- DCSM — Fire Protection and Prevention; DCSM — Hazardous Materials / Chemical Handling; DCSM — Confined Spaces

- DCSM — Blasting; DCSM — Hand Tools and Power Tools; DCSM — Personal Protective Equipment

# 4. Definitions

| Term | Definition |
| --- | --- |
| CHB / SDS | Chemical Hazard Bulletin / (Material) Safety Data Sheet describing a product's hazards and precautions. |
| Airless Spray | High-pressure spray application without compressed air atomisation. |
| Overspray | Paint or coating mist that travels beyond the target surface. |

# 5. Deliverables

- SDS / MSDS log

- Work permits (for spray/coating in restricted areas)

# 6. Flammability Hazards

- Spray painting shall not be performed within 75 ft (23 m) of ignition sources (welding, flame cutting, smoking areas, generators); worksite conditions may justify greater clearances.

- Electrical lighting and power tools/equipment shall be explosion-proof (e.g. UL 844) where solvent vapours are likely; electrical equipment shall be de-energised before coatings are applied to it.

- Solvents and solvent-based coatings shall not be applied to surfaces hotter than 80 °C (176 °F); flammable paints and solvents shall be stored per the Fire Protection chapter, with outside areas free of weeds, debris and combustibles.

- Fire extinguishers shall be available and protected from overspray; work areas shall be cleaned each shift, with paints, solvent cans and rags disposed of in closed, approved containers.

# 7. Health Hazards

- Containers shall carry the proper HazCom label; the CHB/SDS shall be readily available on-site for all coatings, solvents and cleaning fluids, and their precautions followed (see the Hazardous Materials chapter).

- Personnel shall be trained on the product hazards; seal containers when not in use; be provided and trained on the correct respiratory protection (organic-vapour cartridge or supplied-air respirators per the hazard); and wear and inspect the required PPE.

- Adequately ventilate painting/coating areas; use ventilation controls or organic-vapour respirators with toxic-solvent strippers; dispose of coating residue per the Construction Waste Management Plan.

- Wash thoroughly before eating and at end of shift; immediately remove solvent-contaminated clothing (a spontaneous-combustion fire hazard) and discard in closed containers or clean as soon as possible.

# 8. Surface and Worksite Preparation

- Personnel shall be trained on the hazards of the cleaning materials; surfaces not being prepared shall be protected; safe, level work platforms shall be provided; dust/overspray control (e.g. sheeting around blasting) shall minimise impact on adjacent areas.

- Mechanical ventilation shall be provided for confined-space work (see the Confined Spaces chapter); eye and body wash facilities shall be available where chemical solvents/cleaners are used.

- Benzene, gasoline, carbon tetrachloride and chlorinated hydrocarbons shall not be used for cleaning; airless spray equipment shall be grounded; abrasive-blasting equipment shall be bonded and grounded (see the Blasting chapter); power tools shall meet the Hand Tools and Power Tools chapter.

# 9. Application

- Work permits are required for spray painting/coating within a restricted area; pressurised equipment shall be operated and de-energised per the manufacturer's recommendations.

- Spray/airless work areas shall be barricaded against unauthorised entry, with warning signs (e.g. "No entry — spray painting/coating in progress"); precautions shall protect personnel outside the area from vapours.

- Job-site paint/solvent quantities shall be limited to one day's usage, with bulk storage in marked safe areas protected from the sun; portable flammable/combustible-liquid containers shall not exceed 19 L (5 gal) and metal drums 227 L (60 gal).

- Spray guns/nozzles shall not be pointed at any person; spraying shall be performed upwind of the object where practical; chemical solvents shall never be used to clean any part of the body.

# 10. Tarring Operations

- Tar shall be heated at ground level only — never on rooftops or similar unsafe locations — using approved fuel gas in approved cylinders.

- Hot tar shall be transferred by the safest available means (preferably pumped) and never lifted to elevations in buckets.

- Hot-tar operations require, as a minimum, safety shoes, safety glasses, hard hats, face shields, leather gloves, and coveralls with heat-resistant aprons; respiratory protection may be required in confined spaces.

# 11. Records / Related Documents

The SDS log, work permits and training records shall be maintained on-site and archived at project close.

- DCSM — Fire Protection and Prevention

- DCSM — Hazardous Materials / Chemical Handling

- DCSM — Confined Spaces

- DCSM — Personal Protective Equipment

# Revision History

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| R00 | June 2026 | Initial issue. Restructured to the DAN integrated HSSE management-system template and re-branded in accordance with the DAN Brand Book. | Project Director |
| R01 | June 2026 | Re-coded to the updated DAN document-numbering convention (01-000001-000000-DAN-{discipline}-{type}-{seq}). | Project Director |