HEALTH, SAFETY & ENVIRONMENT MANAGEMENT SYSTEM

DAN Construction Safety Manual (DCSM)

Painting and Coating

Document Control

| Document No. | 01-DAN-PRC-HSE-000047-PNTCOT | Revision | R01 |
| --- | --- | --- | --- |
| Issue Date | June 2026 | Classification | Controlled — Internal |
| Document Owner | HSE Department | Page Format | US Letter |

Table of Contents

# 1. Purpose

This document describes the minimum safety requirements for spray painting and coating on DAN projects, covering flammability and health hazards, surface preparation, application and tarring operations.

# 2. Scope

This document applies to all painting and coating activities on DAN projects, including surface preparation, spray and airless application, the use of solvents and cleaners, and hot-tar operations.

# 3. References

- NFPA 30 — Flammable and Combustible Liquids Code; UL 844

- DCSM — Fire Protection and Prevention; DCSM — Hazardous Materials / Chemical Handling; DCSM — Confined Spaces

- DCSM — Blasting; DCSM — Hand Tools and Power Tools; DCSM — Personal Protective Equipment

# 4. Definitions

| Term | Definition |
| --- | --- |
| CHB / SDS | Chemical Hazard Bulletin / (Material) Safety Data Sheet describing a product's hazards and precautions. |
| Airless Spray | High-pressure spray application without compressed air atomisation. |
| Overspray | Paint or coating mist that travels beyond the target surface. |

# 5. Deliverables

- SDS / MSDS log

- Work permits (for spray/coating in restricted areas)

# 6. Flammability Hazards

- Spray painting shall not be performed within 75 ft (23 m) of ignition sources (welding, flame cutting, smoking areas, generators); worksite conditions may justify greater clearances.

- Electrical lighting and power tools/equipment shall be explosion-proof (e.g. UL 844) where solvent vapours are likely; electrical equipment shall be de-energised before coatings are applied to it.

- Solvents and solvent-based coatings shall not be applied to surfaces hotter than 80 °C (176 °F); flammable paints and solvents shall be stored per the Fire Protection chapter, with outside areas free of weeds, debris and combustibles.

- Fire extinguishers shall be available and protected from overspray; work areas shall be cleaned each shift, with paints, solvent cans and rags disposed of in closed, approved containers.

# 7. Health Hazards

- Containers shall carry the proper HazCom label; the CHB/SDS shall be readily available on-site for all coatings, solvents and cleaning fluids, and their precautions followed (see the Hazardous Materials chapter).

- Personnel shall be trained on the product hazards; seal containers when not in use; be provided and trained on the correct respiratory protection (organic-vapour cartridge or supplied-air respirators per the hazard); and wear and inspect the required PPE.

- Adequately ventilate painting/coating areas; use ventilation controls or organic-vapour respirators with toxic-solvent strippers; dispose of coating residue per the Construction Waste Management Plan.

- Wash thoroughly before eating and at end of shift; immediately remove solvent-contaminated clothing (a spontaneous-combustion fire hazard) and discard in closed containers or clean as soon as possible.

# 8. Surface and Worksite Preparation

- Personnel shall be trained on the hazards of the cleaning materials; surfaces not being prepared shall be protected; safe, level work platforms shall be provided; dust/overspray control (e.g. sheeting around blasting) shall minimise impact on adjacent areas.

- Mechanical ventilation shall be provided for confined-space work (see the Confined Spaces chapter); eye and body wash facilities shall be available where chemical solvents/cleaners are used.

- Benzene, gasoline, carbon tetrachloride and chlorinated hydrocarbons shall not be used for cleaning; airless spray equipment shall be grounded; abrasive-blasting equipment shall be bonded and grounded (see the Blasting chapter); power tools shall meet the Hand Tools and Power Tools chapter.

# 9. Application

- Work permits are required for spray painting/coating within a restricted area; pressurised equipment shall be operated and de-energised per the manufacturer's recommendations.

- Spray/airless work areas shall be barricaded against unauthorised entry, with warning signs (e.g. "No entry — spray painting/coating in progress"); precautions shall protect personnel outside the area from vapours.

- Job-site paint/solvent quantities shall be limited to one day's usage, with bulk storage in marked safe areas protected from the sun; portable flammable/combustible-liquid containers shall not exceed 19 L (5 gal) and metal drums 227 L (60 gal).

- Spray guns/nozzles shall not be pointed at any person; spraying shall be performed upwind of the object where practical; chemical solvents shall never be used to clean any part of the body.

# 10. Tarring Operations

- Tar shall be heated at ground level only — never on rooftops or similar unsafe locations — using approved fuel gas in approved cylinders.

- Hot tar shall be transferred by the safest available means (preferably pumped) and never lifted to elevations in buckets.

- Hot-tar operations require, as a minimum, safety shoes, safety glasses, hard hats, face shields, leather gloves, and coveralls with heat-resistant aprons; respiratory protection may be required in confined spaces.

# 11. Records / Related Documents

The SDS log, work permits and training records shall be maintained on-site and archived at project close.

- DCSM — Fire Protection and Prevention

- DCSM — Hazardous Materials / Chemical Handling

- DCSM — Confined Spaces

- DCSM — Personal Protective Equipment

# Revision History

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| R00 | June 2026 | Initial issue. Restructured to the DAN integrated HSSE management-system template and re-branded in accordance with the DAN Brand Book. | Project Director |
| R01 | June 2026 | Re-coded to the updated DAN document-numbering convention (01-DAN-PRC-HSE-{seq}{code}). | Project Director |