HEALTH, SAFETY & ENVIRONMENT MANAGEMENT SYSTEM

DAN HSE Minimum Requirements

General Site Safety Rules

Document Control

| Document No. | 01-000001-000000-DAN-HSE-MNR-000002 | Revision | R01 |
| --- | --- | --- | --- |
| Issue Date | June 2026 | Classification | Controlled — Internal |
| Document Owner | HSSE Department | Page Format | US Letter |

Table of Contents

Purpose

Scope

Definitions

1. Logistics – deliveries and storage

2. Workers’ Transportation

3. Housekeeping

4. Lighting

5. Temporary work

Standards

Tools and References

Revision History

# Purpose

The purpose of General Site Safety Rules HSE requirement is to provide efficient and coordinated logistics services, to ensure an uninterrupted and safe storage process to allow safe movement around the stored materials to prevent the risk of slip, trip, fall, injury, and property damage.

# Scope

This HSE minimum requirement applies to all parties’ personnel working within DAN Company Company Limited “DAN” projects, site offices, or DAN-controlled work areas.

# Definitions

| Term | Definition |
| --- | --- |
| Logistic | The overall process of managing how resources are acquired, stored, and transported |
| Coupling device | The method is used to connect two shafts at their ends to transmit power. |
| Uncoupling | The act or process of detaching |
| Worker transportation | A motor vehicle that is provided by or arranged by an employer to transport 3 or more workers to and or from a workplace. |
| Glare | A visual sensation caused by excessive and uncontrolled brightness |
| Strobing effects | Producing very short and intense flashes |
| Led | Light-emitting diode |
| Lux level | The si unit of illuminance. One lux is one lumen per square meter (lm/m2) |

# 1. Logistics – deliveries and storage

- Material deliveries/collection processes will be established to ensure:

- All drivers report to security upon arrival

- Drivers are given an HSE briefing/induction before entry onto the site.

- Delivery/collection vehicles are given instructions to reach storage/collection areas.

- Vehicle loads are checked before entering the site.

- Vehicle loads are checked and approved before exiting the site.

- Uncoupling/coupling of trailers takes place in a designated area.

- Storage areas will be:

- Of suitable size with space to allow safe movement around the stored materials.

- Located to minimize disruption to construction activities.

- Located away from emergency routes.

- Managed by a responsible person [logistics manager]

- Storage areas will be considered controlled work environments, this includes:

- Clear demarcation, secure barriers, sign-posted and adequate lighting

- Compacted hard standing - concrete hard standing for palletized materials

- Specified limits on pallet stacks to prevent toppling/overturning

- Safe unloading/loading methods - controls for working at height

- Use of storage racks will require inspections and mark-up with safe working limits [swl]

- Equipped with emergency resources including proper backup generator, fire detection, alarms, and extinguishers.

- Materials must not be stored outside the designated areas

- Material storage areas must not be used by workers as alternative rest areas

- Material storage areas will be:

- Visually inspected daily

- Inspected weekly and the outcomes documented

# 2. Workers’ Transportation

- Employer-provided transportation will meet the following requirements:

- Compliant with Ministry of Health seating and occupancy requirements

- All vehicles will be maintained and remain safe for use

- Vehicles used will be subject to an annual inspection by the competent local authority

- Drivers will be licensed by road transport authority requirements.

- Vehicle’s safety controls for the driver and all passengers and the wellbeing provisions will remain operational, including;

- i. Air-conditioning,

- ii. Safety controls [seat belts and handgrips],

- iii. Emergency arrangements [first-aid kits, fire extinguishers, emergency exit windows, emergency lighting, and signage]

- Workers’ transportation will be frequently inspected by a competent person and the outcomes documented

# 3. Housekeeping

All workplaces shall be kept free from safety and health hazards.

- It is everyone’s responsibility to make sure housekeeping is a daily activity.

- Good housekeeping will be the norm and there are no acceptable excuses for poor housekeeping

- All places of work will be kept clean, orderly, and in a sanitary condition to prevent injury, harm, and disease

- The floor of every workplace will be maintained and kept in a clean and dry condition to prevent slips, trips, and falls.

- Workplaces will be kept free from biological and chemical hazards.

- Use of hazardous materials must only be carried out using an approved MSRA

- Workplaces will be kept free from excessive storage of materials and waste to prevent fire hazards and public health issues

- Workplaces will be kept clear of dust accumulation to protect occupational health

- Waste stockpiling must only be permitted in approved locations

- Material stockpiling must only be permitted in approved locations

- All tools and portal equipment will be removed and stored in approved locations

# 4. Lighting

It is highly recommended and required, whenever applicable, to install and use Lighting bulbs that don’t produce heat, E.g. LEDs or light-emitting diodes don’t require any heat to help their elements ‘glow’.

- All areas of construction must be well-lit, either with natural or artificial lighting.

- Where required appropriate artificial lighting levels will be provided as follows:

| Location / area | Minimum lux level |
| --- | --- |
| Project offices | 323 |
| First-aid stations / medical treatment centres | 323 |
| General work areas | 323 |
| Welfare facilities | 108 |
| Mechanical and electrical equipment rooms | 108 |
| General construction plant and shops | 108 |
| General access / egress routes | 54 |
| Loading platforms | 54 |
| Storage areas | 54 |
| Warehouse | 54 |
| Maintenance areas | 54 |
| General construction sites | 54 |
| Concrete placement | 54 |
| Excavations, tunnels, and shafts | 54 |
| Emergency routes | 54 |

- Artificial lighting sources will be:

- Located to prevent glare and shadows

- Maintained to prevent strobing effects

- Portable task lighting equipment will use LED bulbs only

- Temporary artificial lighting will be:

- Equipped with guards to prevent accidental contact

- Fastened securely to prevent contact with combustible materials

- Powered using a heavy-duty electrical feed cord

- Suspended using appropriate fixings – not the electrical feed cord

- Artificial lighting sources will be inspected, tested, and maintained, and outcomes recorded.

- Periodic lighting surveys will be carried out and outcomes recorded

# 5. Temporary work

The Design of temporary structures must be reviewed and approved through the CSC by a chartered Structural Engineer.

- Temporary works will apply to the following operations, but are not limited to:

- Propping/back propping/formwork/steelwork erection

- Crane positioning/tower crane base siting

- Temporary access ramps

- Working platforms/loading platforms / non-standard scaffold configurations

- Security hoardings/project signposts

- Excavations/excavation shoring systems/sheet pile walls

- All persons involved in managing temporary works:

- Must be appointed in writing by the employer, these include:

- i. Designers/engineers

- ii. Temporary works co-ordinator

- iii. Temporary work supervisors

- Provided with specific competency-based training

- Awareness briefings will be provided to those involved in using temporary works to recognize and respond to the hazards.

- Temporary work site operations will have the following in place:

- Defined roles & responsibilities

- Temporary works checklist

- inspection records

- PTW load & dismantle

- Temporary work site operations will be: approved design

- Frequent inspection/Audit Records and evidence

- Approved logistic plan

- Approved project specific HSE plan procedures

- Induction & security briefing record

- Security records/briefings & inductions

- Vehicle load checklist

- Workplace inspection records

- Lighting plan [part of the logistics plan]

- Lighting survey reports

- Site inspection records

- PTW

# Standards

- DAN Construction HSE Management System Manual (01-000001-000000-DAN-HSE-MAN-000001) and management procedures

- Saudi labor law – royal decree no. M/51, 23 Sha’ban 1426 / 27 September 2005

# Tools and References

- Osha regulations 1926.56 illumination

- Osha regulations 1915.82 lighting

- Osha regulations 1915.81 housekeeping

# Revision History

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| R01 | June 2026 | First issue — DAN HSE Minimum Requirement, adapted from the source OH&S minimum requirement, re-branded to DAN and coded to the DAN document-numbering convention (01-000001-000000-DAN-HSE-MNR-{seq}). | HSSE Manager |